MD-01 光电二极管偏压DC-DC选型报告

- 型号

MAX5026EUT+T

- 价格与供货情况

立创商城 C87603 10+: ￥8.92 100+: ￥6.49

- 关键参数

- Input Voltage Range: 3V to 11V

- Wide Output Voltage Range: VCC to 36V

- Output Power: 120mW (max)

- User-Adjustable Output Voltage with Using External Feedback Resistors

- Internal 1.3Ω (typ), 40V Switch

- Constant PWM Frequency Provides Easy Filtering in Low-Noise Applications

- 500kHz (typ) Switching Frequency

- 1μA (max) Shutdown Current

- Small, 6-Pin SOT23 Package

- 主要原因

- 供电电压范围

3V to 11V范围满足3.3V/5V供电要求。

- 宽电压输出范围: VCC to 36V

可同时满足PIN和APD的偏压需求（３倍压后偏压>100V）。

- 固定和较低的开关频率

这两个特性使之适合为光电二极管提供偏压。

- 封装

提供了SOT23-5紧凑型封装，为后续更严格空间限制的Layout提供可能性。