MD-01 参考电压缓冲运放选型报告

- 型号

TP5551-TR

- 价格与供货情况

官网https://www.3peakic.cn/quickBuy 20~1000: ￥4.92

- 关键参数

- Supply Voltage: 1.8 V to 5.5 V

- Low Offset Voltage: 5 μV (max)

- Zero Drift: 0.05 μV/°C (max)

- No 1/f Noise Corner Down to 0.1Hz

Input Noise Voltage: 15 nV/√Hz at 1 kHz

0.1 Hz to 10 Hz Voltage Noise: 350 nVPP

- Slew Rate: 2.5 V/μs

- Bandwidth: 3.5 MHz

- Low Supply Current: 550 μA per Amplifier

- Low Input Bias Current: 50 pA Typical

- Rail-to-Rail Output Voltage Range

- High gain, CMRR, PSRR: 130 dB

- 7 kV HBM ESD Rating

- Operating Temperature Range: –40°C to 125°C

- 主要原因

- 供电电压范围

1.8V~5.5V范围满足3.3V供电要求。

- 轨到轨的输出

采用3.3V供电后，信号范围被压缩，轨到轨的输出使得输出范围尽量大。

- 1/f噪声

1/f噪声转折频率低至0.1Hz，符合低频电压噪声的标准规格。

- 低噪声和零漂

这两个特性使该运放适合电压参考的缓冲。

- 封装

提供了SOT23-5紧凑型封装，为后续更严格空间限制的Layout提供可能性。