MD-01 激光二极管电源DC-DC选型报告

- 型号

SGM6602-12YTDI6G/TR

- 价格与供货情况

代理 单价1.3含税 最小包装3K 交期4-6周

- 关键参数

- 1.8V to 5.5V Operating Input Voltage Range

- Up to 20V Output Voltage

- Up to 85% Efficiency at 3.6V Input and 12V Output

- 900mA (TYP) Switch Current

- Internal 5ms Soft-Start Time

- Less than 1μA Ultra-Low Shutdown Current

- Integrated Isolation Switch

- Power-Save Mode at Light Load

- Input and Output are Disconnected during Shutdown

- Output Short-Circuit and Over-Voltage Protections

- Thermal Shutdown

- Available in Green WLCSP-0.8×1.2-6B and TDFN-2×2-6AL Packages

- 主要原因

- 供电电压范围

1.8V~5.5V范围满足3.3V/5V供电要求。

- 高达20V的输出范围

满足脉冲激光的驱动需求。

- 同步Boost

更小的电感电容，占PCB面积更小。

- 封装

提供了WLCSP/ TDFN紧凑型封装，为后续更严格空间限制的Layout提供可能性。